SCHEDULE - E

(See Clauses 2.1 and 14.2)

MAINTENANCE REQUIREMENTS

## Maintenance Requirements

## 1.1	The Contractor shall, at all times maintain the Project Highway in accordance with the provisions of this Agreement, Applicable Laws and Applicable Permits.

## 1.2	The Contractor shall repair or rectify any Defect or deficiency set forth in Paragraph 2 of this Schedule-E within the time limit specified therein and any failure in this behalf shall constitute non-fulfillment of the Maintenance obligations by the Contractor. Upon occurrence of any breach hereunder, the Authority shall be entitled to effect reduction in monthly lump sum payment as set forth in Clause 14.6 of this Agreement, without prejudice to the rights of the Authority under this Agreement, including Termination thereof.

## 1.3	All Materials, works and construction operations shall conform to the MORTH Specifications for Road and Bridge Works, and the relevant IRC publications. Where the specifications for a work are not given, Good Industry Practice shall be adopted.

- Repair/Rectification of Defects and Deficiencies

The obligations of the Contractor in respect of Maintenance Requirements shall include repair and rectification of the defects and deficiencies specified in Annex - I of this Schedule-E within the time limit set forth therein.

## 3	Other Defects and Deficiencies

## In respect of any defect or deficiency not specified in Annex - I of this Schedule-E, the Authority’s Engineer may, in conformity with Good Industry Practice, specify the permissible limit of deviation or deterioration with reference to the Specifications and Standards, and any deviation or deterioration beyond the permissible limit shall be repaired or rectified by the Contractor within the time limit specified by the Authority’s Engineer.

## 4	Extension of Time Limit

## Notwithstanding anything to the contrary specified in this Schedule-E, if the nature and extent of any defect or deficiency justifies more time for its repair or rectification than the time specified herein, the Contractor shall be entitled to additional time in conformity with Good Industry Practice. Such additional time shall be determined by the Authority’s Engineer and conveyed to the Contractor and the Authority with reasons thereof.

## 5	Emergency Repairs/Restoration

## Notwithstanding anything to the contrary contained in this Schedule-E, if any defect, deficiency or deterioration in the Project Highway poses a hazard to safety or risk of damage to property, the Contractor shall promptly take all reasonable measures for eliminating or minimizing such danger.

## 6	Daily inspection by the Contractor

## The Contractor shall, through its engineer, undertake a daily visual inspection of the Project Highway and maintain a record thereof in a register to be kept in such form and manner as the Authority’s Engineer may specify. Such record shall be kept in safe custody of the Contractor and shall be open to inspection by the Authority and the Authority’s Engineer at any time during office hours.

7.	Pre-monsoon inspection / Post-monsoon inspection

The Contractor shall carry out a detailed pre-monsoon inspection of all bridges, culverts and drainage system before [1st June] every year in accordance with the guidelines contained in IRC: SP35. Report of this inspection together with details of proposed maintenance works as required on the basis of this inspection shall be sent to the Authority’s Engineer before the 10th June every year. The Contractor shall complete the required repairs before the onset of the monsoon and send to the Authority’s Engineer a compliance report. Post monsoon inspection shall be done by the 30th September and the inspection report together with details of any damages observed and proposed action to remedy the same shall be sent to the Authority’s Engineer.

8.	Repairs on account of natural calamities

All damages occurring to the Project Highway on account of a Force Majeure Event or default or neglect of the Authority shall be undertaken by the Authority at its own cost. The Authority may instruct the Contractor to undertake the repairs at the rates agreed between the Parties.

Annex - I

(Schedule-E)

# Repair/Rectification of Defects and Deficiencies

The Contractor shall repair and rectify the Defects and deficiencies specified in this Annex-I of Schedule-E within the time limit set forth in the table below.

| Nature of Defect or Deficiency |  | Time limit for Repair/Rectification |
| --- | --- | --- |
| ROADS |  |  |
| (a) | Carriageway and paved shoulders |  |
| (i) | Breach or blockade | Temporary restoration of traffic within 24 hours; permanent restoration within 15 (fifteen) days |
| (ii) | Roughness value exceeding 2,200 mm in a stretch of 1 km (as measured by a calibrated bump integrator) | 120 (one hundred and twenty) days |
| (iii) | Pot holes | 24 hours |
| (iv) | Any cracks in road surface | 15 (fifteen) days |
| (v) | Any depressions, rutting exceeding 10 mm in road surface | 30 (thirty) days |
| (vi) | Bleeding/skidding | 7 (seven) days |
| (vii) | Any other defect/distress on the road | 15 (fifteen) days |
| (viii) | Damage to pavement edges | 15 (fifteen) days |
| (ix) | Removal of debris, dead animals | 6 hours |
| (b) | Granular earth shoulders, side slopes, drains and culverts |  |
| (i) | Variation by more than 1 % in the prescribed slope of camber/cross fall (shall not be less than the camber on the main carriageway) | 7 (seven) days |
| (ii) | Edge drop at shoulders exceeding 40 mm | 7 (seven) days |
| (iii) | Variation by more than 15% in the prescribed side (embankment) slopes | 30 (thirty) days |
| (iv) | Rain cuts/gullies in slope | 7 (seven) days |
| (v) | Damage to or silting of culverts and side drains | 7 (seven) days |
| (vi) | Desilting of drains in urban/semi-urban areas | 24 hours |
| (vii) | Railing, parapets, crash barriers | 7 (seven) days (Restore immediately if causing safety hazard) |
| (c) | Road side furniture including road sign and pavement marking |  |
| (i) | Damage to shape or position, poor visibility or loss of retro-reflectivity | 48 hours |
| (ii) | Painting of km stone, railing, parapets, crash barriers | As and when required/Once every year |
| (iii) | Damaged/missing road signs requiring replacement | 7 (seven) days |
| (iv) | Damage to road mark ups | 7 (seven) days |
| (d) | Road lighting |  |
| (i) | Any major failure of the system | 24 hours |
| (ii) | Faults and minor failures | 8 hours |
| (e) | Trees and plantation |  |
| (i) | Obstruction in a minimum head-room of 5 m above carriageway or obstruction in visibility of road signs | 24 hours |
| (ii) | Removal of fallen trees from carriageway | 4 hours |
| (iii) | Deterioration in health of trees and bushes | Timely watering and treatment |
| (iv) | Trees and bushes requiring replacement | 30 (thirty) days |
| (v) | Removal of vegetation affecting sight line and road structures | 15 (fifteen) days |
| (f) | Rest Area |  |
| (i) | Cleaning of toilets | Every 4 hours |
| (ii) | Defects in electrical, water and sanitary installations | 24 hours |
| (g) | Toll Plazas |  |
| (h) | Other Project Facilities and Approach roads |  |
| (i) | Damage in approach roads, pedestrian facilities, truck lay-byes, bus-bays, bus-shelters, cattle crossings, [Traffic Aid Posts, Medical Aid Posts] and service roads | 15 (fifteen) days |
| (ii) | Damaged vehicles or debris on the road | 4 (four) hours |
| (iii) | Malfunctioning of the mobile crane | 4 (four) hours |
| BRIDGES |  |  |
| (a) | Superstructure |  |
| (i) | Any damage, cracks, spalling/ scaling Temporary measures Permanent measures | within 48 hours within 15 (fifteen) days or as specified by the Authority’s Engineer |
| (b) | Foundations |  |
| (i) | Scouring and/or cavitation | 15 (fifteen) days |
| (c) | Piers, abutments, return walls and wing walls |  |
| (i) | Cracks and damages including settlement and tilting, spalling, scaling | 30 (thirty) days |
| (d) | Bearings (metallic) of bridges |  |
| (i) | Deformation, damages, tilting or shifting of bearings | 15 (fifteen) days Greasing of metallic bearings once in a year |
| (e) | Joints |  |
| (i) | Malfunctioning of joints | 15 (fifteen) days |
| (f) | Other items |  |
| (i) | Deforming of pads in elastomeric bearings | 7 (seven) days |
| (ii) | Gathering of dirt in bearings and joints; or clogging of spouts, weep holes and vent-holes | 3 (three) days |
| (iii) | Damage or deterioration in kerbs, parapets, handrails and crash barriers | 3 (three) days (immediately within 24 hours if posing danger to safety) |
| (iv) | Rain-cuts or erosion of banks of the side slopes of approaches | 7 (seven) days |
| (v) | Damage to wearing coat | 15 (fifteen) days |
| (vi) | Damage or deterioration in approach slabs, pitching, apron, toes, floor or guide bunds | 30 (thirty) days |
| (vii) | Growth of vegetation affecting the structure or obstructing the waterway | 15 (fifteen) days |
| (g) | Hill Roads |  |
| (i) | Damage to retaining wall/breast wall | 7 (seven) days |
| (ii) | Landslides requiring clearance | 12 (twelve) hours |
| (iii) | Snow requiring clearance | 24 (twenty four) hours |

[Note: Where necessary, the Authority may modify the time limit for repair/rectification, or add to the nature of defect or deficiency before issuing the bidding document, with the approval of the competent authority.]