Schedule-Q

(SeeClause14.10)

Tests on Completion of Construction

- Riding Quality test:

Riding quality test: Riding quality of each lane of the carriageway shall be checked

with the  help  of  a  calibrated  bump  integrator  and  the  maximum  permissible

roughness  for purposes of this Test shall be [2,500 (two thousand five hundred)] mm

for each kilometer.

2. Visual and physical test:

The  Authority’s  Engineer  shall  conduct  a  visual  and  physical  check  of

construction  to determine that all works and equipment forming part thereof conform

to the provisions of this  Agreement.  The  physical  tests  shall  include  measurement

of  cracking,  rutting, stripping and potholes and the permissible values are given

below: • Area of cracking not more than 2 % area

• Area of rutting with rut depth more than 10 mm - not more than 1 …. % area

• Area of stripping: not more than 2 % area

• Area of potholes: Nil

• Edge drop – Shall not be more than 15 mm