Annexure-3

- The details as required are given under:-

- Details of approach Road: length of approach road as under:-

(i) On western Portal	-	5.025 km

(ii)On Eastern Portal	-	0.803 km

The layout drawing L Section and cross section are enclosed as per Appx. – ‘A’

- Details of Bridges:	Permanent bridge span of 60 mtr. At Km. 4.475 on approach on Western Portal has been proposed. The GAD is enclosed as Appx. – ‘B’

- Details of Snow Galleries & Avalanche Control Structure: Details of Various structures/ techniques recommended by SASE are as under:-

- At Km. 2.850 on approach road to Western Portal, there is 700 mt. long avalanche path which is a deep nallah in upper portion and fans out near the existing highway. 150 mt. long snow gallery is proposed at this location with suitable wing walls at both ends of the gallery to guide the avalanche flow over its length only. Structural design considerations, material specifications and other details have been given in the study report.

- At avalanche site SL 40, where the affected length of road is 600m each (existing and new) SASE recommends “OMBRELLO” (Umbrella) type snow retaining structures.

- At avalanche  sites SL 41 and 42, the existing road is in the track zone of the two avalanches. SASE recommends provision of retarding mounds of suitable geometry in this location.

- Avalanche site SL 43 is a major and frequent avalanche site and has a tendency to fan out in the run out zone. SASE recommends a combination of mounds and catch dam for this site. A catch dam of 350m length and three rows of mounds will be required to prevent avalanche debris from reaching the approach road. 21 mounds will be required. The height of mounds will be 7 m and height of catch dam will be 8 m on upstream side.

- Avalanche site SL 44 is the biggest avalanche site on this stretch and has as huge catchment area and large run out zone. The affected road length is 700m. SASE recommends combination of mounds and catch dam exactly similar to that at SL 43.

The detailed report of SASE is enclosed as per Appx ‘C’.

- Details of culverts:	Details of culverts, MCBC and permanent structure enclosed as per Appx – ‘D’.

- Requirement of Amarnath Shrine Board:	The details of work required by SASB are as under:-

| S.No. | Facility/ Description | Rs. (in Lacs) | To be constructed by |
| --- | --- | --- | --- |
|  | Erection of toilet/ bath units and laying of sewer line | 25.00 | SASB |
|  | Erection of prefabricated Huts - Medical Hut - Air Traffic Control (ATC HUT) - Disaster Management Hut - PHE Hut - BSF Hut - Army Hut | 57.00 | SASB |
|  | Chain link fencing all around the Baltal Camp | 98.40 | SASB |
|  | Clock Room | 1.00 | SASB |
|  | Layout of Tented Area, including Police Tented Accommodation and JPCR | 2.00 | SASB |
|  | Construction of Main Entrance Gate and 02 Exit Gates | 2.00 | SASB |
|  | Levelling of the area and construction of drains | 30.00 | SASB |
|  | Total | 215.40 |  |

The revised cost for shifting of utilities of various departments has been projected by DC Ganderbal for Rs. 10.00 crores including Rs. 215.40 lacs of SASB work vide their letter No. DCG/SQ/109-14 dated 17 April, 2014. The site plan and sketch map profile of road network is enclosed at Appx – ‘E’

- Present position of LA/FC:	The land required for Zozila Tunnel approach road are as under:-

| S.No. | Type of land | Total Area | Area required | Remarks |
| --- | --- | --- | --- | --- |
| (i) | Forest land | 208.88 HA | 97.82% | Payment for Rs. 169191042 paid on 31st March, 2014 to PCCF |
| (ii) | Revenue land | 4.60 HA | 2.18 | Payment for Rs. 39372000 paid on 31st March 2014 to DC Ganderbal |

Indent for service road to connect Amarnath Road is 2.3 Km and link road to Amarnath Base Camp 0.4 Km. total Area 6.48 HA was placed with forest authority by HQ CE (P) Beacon Letter No. 200093/LAFC/Amarnath/SS/02/E2 Wks dated 9th October, 2014. However, final alignment of service road is still not finalized by State Government and SASB in consultation with rep. MoRTH.

- Details of nearest Power Station:	The nearest power station is in upper Sindh Project Phase-I at Sumbal at Km. 57.00 on SSG road.

- Maintenance of Existing Roads:	The MoU between BRO and executing agency should be signed for maintenance of SSG road which will be damaged due to movement of heavy loaded vehicles during the construction of Zozilla Tunnel.